ELEC2003.3

Digital filters

Dmitry Ignatyev

27 October 2009

Abstract

This report examines methods of design and implementation of digital filters in context of a distorted satellite’s signal filtering.

Background information on sampling of signals, aliasing and z-domain is presented. Examples of various design methods are discussed, taking an analogue notch filter as a base. Approximation of derivatives and z-transform is used to design an equivalent digital filter, followed by bilinear transformation. Relevance of impulse response, evaluation of frequency response and the link between them are considered. Finite and infinite impulse response filters (FIR and IIR) are discussed.

All-digital design methods are shown, using Yule-Walker algorithm and direct pole-zero mapping. Relevant Matlab commands and code examples are included throughout.

Finally, methods of cascading digital filters and increasing their order are shown. As an example, a cascaded notch and low-pass filter is designed step-by-step.

Report also looks at batch filtering, which requires the entire signal to be sampled and stored prior to processing.

Contents

1	Introduction	3

2	Theory	3

1	Experimental procedure	3

2	Getting started	3

2.1	The signals to be filtered	3

2.2	Sampling of signals	4

3	The Z-notation	5

3.1	The Z-transform	5

4	Digital filters from analogue design	6

4.1	An analogue filter for 0.1Hz disturbance	6

4.2	A digital filter by numerical differentiation	10

4.3	Digital Filters by s-plane to z-plane mappings	13

5	IMPULSE AND FREQUENCY RESPONSES OF DIGITAL FILTERS	14

5.1	Impulse Response of a Digital Filter	14

5.2	FIR and IIR Filters	14

5.3	Frequency Response of a Digital Filter	15

6	DIRECT DIGITAL FILTER DESIGN	16

6.1	Using the Yulewalk function	16

6.2	Direct approach	18

6.3	Cascade approach	20

7	Optional section	21

8	Batch filtering	27

9	Discussion	28

10	Conclusions	29

11	References	29

# Introduction

# Getting started

## The signals to be filtered

Figure 2-1 shows an example of a satellite’s position signal in its ideal form.

Figure 2-1

However in practice this signal may be distorted. In our case, it is modulated by vibration of the satellite at 0.1Hz. Figure 2-2 shows how distorted signal might look like.

Figure 2-2

The signal is sampled at sample intervals of 0.5s or sampling frequency of 2Hz. Therefore 128 samples of the signal occupy space of 64 seconds. Time vector t is generated with Matlab code as follows:

t=(0:127)*64/128;

It is worth noting that unlike most programming languages, arrays in Matlab start at index 1.  So in array like t, we cannot access 0th element, but we can access the Nth one, where N is the size of the array (vector):

>> t(128)

ans =

63.5000

## Sampling of signals

Sampling rate is a very important consideration when designing digital filters. Nyquist theorem states that sampling frequency must be at least twice as high as the highest frequency component in the sampled signal. If this is not obeyed, aliasing occurs. Figure 2-3 shows 2 signals sampled at 2Hz. The 0.6Hz signal is below the Nyquist frequency which is 1Hz, and is sampled correctly. 1.4Hz signal is higher than Nyquist frequency, and when sampled it “looks” like the 0.6Hz signal, which is of course wrong. This means that digital filters repeat their frequency response for frequencies higher than half the sampling rate.

Figure 2-3

Code below shows that 0.6Hz and 1.4Hz signals “look” the same when sampled at 2Hz:

z1 = cos(2*pi*0.6*t);

z2 = cos(2*pi*1.4*t);

>> z1(1:10)

1.0000 -0.3090 -0.8090 0.8090 0.3090 -1.0000 0.3090 0.8090 -0.8090 -0.3090

>> z2(1:10)

- -0.3090 -0.8090 0.8090 0.3090 -1.0000 0.3090 0.8090 -0.8090 -0.3090

# The Z-notation

## The Z-transform

Z transform is defined as:

| Yz=Zyk=n=0n=∞ynz-n | Equation 3-1 |
| --- | --- |

It can be used to transform various functions into z-domain, such as exponentials.

>> y=exp(-t/30);

>> plot(t,y,'.');

y(2)/y(1) = 0.9835

y(3)/y(2) = 0.9835

Therefore signal y[k]=exp(-k/30) can be represented as  yk=1×{1,0.98351,0.98352,…}

Applying Z-transform to this sequence we get

Yz=y01+az-1+a2z-2+an-1z-n-1

If we notice that this is the binomial expansion of 1-az-1-1, where x=-az-1, n=-1

1+xn=1+nx1!+nn-1x22!+…

We can represent it as y0zz-a=zz-0.9835

The value of this function is infinity when z=0.9835, in other words it has a pole at this value.

Operator z-1 is also a delay operator. That is:

Zyk-1=z-1×Z(yk)

Generally, az-n means multiplying sample x[c-n] by constant value a. Sample x[c] in this sense is the current, or the most recent sample.

# Digital filters from analogue design

## An analogue filter for 0.1Hz disturbance

Figure 4-1 shows an analogue notch filter.

Figure 4-1

To have a notch in frequency response at a particular frequency, the combined impedance of L and C must tend to infinity at that frequency. Thus:

ZT=jωL∥1jωC=jωLjωCj2ω2LC+1jωC=jωL-ω2LC+1

This will become infinity when ω2LC=1.

Another way to analyze this circuit is to derive the governing differential equation:

| d2ydt2LC+dydt+y=d2udt2LC+u | Equation 4-1 |
| --- | --- |

Using Laplace transform, it can be converted into s-domain as follows:

Ldydt=sYs-y(0)

Ld2ydt2=s2Ys-sy0-y'(0)

y0=y'0=0

s2YsLC+sYsLR+Ys=s2UsLC+U(s)

Yss2LC+sLR+1=Uss2LC+1

| Y(s)U(s)=s2LC+1s2LC+sLR+1 | Equation 4-2 |
| --- | --- |

We can analyze the s-plane equation. A zero would correspond to the notch in the frequency response, thus we can work out values of L and C. If we want the notch to be at 0.1Hz, corresponding to satellite’s frequency of vibrations, we get:

s2LC+1=0

s2LC=-1

s=jω

(jω)2LC=-1

ω=0.1Hz×2π=0.2π

-0.04π2LC=-1

LC=10.04π2≈2.53

Reassuringly it gives the same result as the impedance analysis. For simplicity, we take value of L as 1H and value of R as 1Ω.

We can now analyze the frequency response of this filter. Matlab command freqs does that by taking s-domain polynomial coefficients and substituting s=jω.

f = (0:127)*2/128; % 128 frequencies from 0 to 2Hz

w = 2*pi*f;

const = (10/(2*pi))^2; %=2.53

num=[const 0 1];

den=[const 1 1];

cfr=freqs(num,den,w);

mod=abs(cfr); % Taking magnitude of complex frequency response

arg=angle(cfr)*180/pi; % Taking phase and converting it to degrees

plot(f,mod)

plot(f,arg)

Figure 4-2 shows the resulting frequency response plot. There is a sharp notch at 0.1Hz and rapid phase change around that frequency.

Figure 4-2

Figure 4-3

I chose to show poles and zeros of the transfer function in Cartesian form in Figure 4-3. The zero on the imaginary axis at 0.6 rad/sec corresponds to the notch at w/2π=0.1Hz.

Because we already have the complex frequency response vector cfr, we can convert signal1 into frequency domain too, and multiply 2 vectors together element-by-element to get the frequency spectrum of the output signal. We can then convert it back to time domain using the ifft command.

afsig=ifft(cfr.*fft(signal1,128));

mx=abs(afsig1);

Figure 4-4 shows signal1 before and after filtering. It is clear that 0.1Hz component has been greatly reduced in the output signal.

Figure 4-4

## A digital filter by numerical differentiation

We can approximate linear differentiation for sampled data. The definition of differentiation is just “the rate of change”. In case of digitally sampled date, it is the difference between the values of neighbouring samples.

Figure 4-5

Figure 4-5 illustrates that good approximation is achieved by so called central difference:

| dydt≈yn+1-yn-12T | Equation 4-3 |
| --- | --- |

We can also find the second derivative. It is defined as “rate of change of change”. We can calculate that if we take the forward difference and backward difference, and find the rate of change between them:

| yn+1-ynT-yn-yn-1TT=yn+1-2yn+yn-1T2 | Equation 4-4 |
| --- | --- |

We can substitute Equation 4-3 and Equation 4-4 into Equation 3-1:

Because we arbitrarily chose both L and R to be 1, the term L/R becomes just 1 and can be omitted from further calculations. Obviously this will seldom be the case, and we have to account for this term. Expanding out all the terms and grouping by yn-1, yn and yn+1 we get

Matlab function filter implements the following algorithm:

a(1)*y(n) = b(1)*x(n) + b(2)*x(n-1) + ... + b(nb+1)*x(n-nb) - a(2)*y(n-1) - ... - a(na+1)*y(n-na)

We can see that b coefficients can be calculated as un factors, and a coefficients a as yn factors. This is the standard notation – input is multiplied by factors b, and, for recursive filters, output is multiplied by factors a.

T=0.5 and T2 is shorthand for T2:

>> a=[(LC/T2+1/(2*T)) (1-2*LC/T2) (LC/T2-1/(2*T))]

a =

11.1321 -19.2642 9.1321

>> b=[(LC/T2) (1-2*LC/T2) (LC/T2)]

b =

10.1321 -19.2642 10.1321

Because a1 is not equal to 1, all the coefficients will be divided by a1 (normalised).

Figure 4-6 shows the spectrum of signal1 before and after filtering with filter command. It is clear that the unwanted 0.1Hz component has been almost completely removed.

Figure 4-6

It is informative to plot the roots of the z-domain transfer function.

zzn=roots(b);

pzn=roots(a);

polar(0,1,’ ’)

hold

polar(atan2(imag(zzn),real(zzn)),abs(zzn),’o’)

polar(atan2(imag(pzn),real(pzn)),abs(pzn),’*’)

Figure 4-7 shows the polar plot of zeros and poles. We can see that there is a zero right on the unit circle, which corresponds to the notch in the frequency response.

Figure 4-7

## Digital Filters by s-plane to z-plane mappings

We can transform s-domain equations directly into z-domain:

and

s24T2z2-2z+1z2+2z+1

Substituting these into Equation 4-2 we get:

Using simple Maple script to simplify the equation and group it by powers of z, we can then divide the whole equation by z2, to get it in the standard form, and extract coefficients a and b. Values of R and L are omitted because they are equal to 1.

b=

| >> 4*LC+T2 10.3821 | >> 2*T2-8*LC -19.7642 | >> 4*LC+T2 10.3821 |
| --- | --- | --- |

a=

| >> 4*LC+2*T+T2 11.3821 | >> -8*LC+2*T2 -19.7642 | >> 4*LC-2*L*T+T2 9.3821 |
| --- | --- | --- |

We can see immediately that these coefficients match the ones obtained through the difference equation exactly, which proves that bilinear transform was performed correctly.

Matlab has a built-in command to implement bilinear transform, bilinear. It takes the polynomial coefficients of s-place transfer function and gives the z-domain transfer function coefficients in descending powers of z, taking sampling frequency into account. Again, it gives the same values, normalised by a1.

>> [numd,dend]=bilinear([LC 0 1],[LC 1 1],2);

numd =

0.9121   -1.7364    0.9121

dend =

1.0000   -1.7364    0.8243

# IMPULSE AND FREQUENCY RESPONSES OF DIGITAL FILTERS

## Impulse Response of a Digital Filter

Impulse response is an important characteristic of a digital filter. In analogue domain impulse is approximated by a delta function δ(t), in digital domain impulse is a “1” followed by all the “zeros”. Such sequence effectively presents all the frequencies to the digital filter at once, so spectrum of impulse response is filter’s frequency response. It is the result of the convolution theorem: convoluting two signals in time domain is the same as multiplying them in frequency domain.

>> u=[1 zeros(1, 127)];

>> impl=filter(b,a,u);

>> plot(t,impl,'*');

>> plot(fr,ampl(fft(impl)));

Figure 5-1

## FIR and IIR Filters

Digital filters are divided into two categories: Finite Impulse Response (FIR) and Infinite Impulse Response (IIR). The former category only takes the input into account, and is always stable. The output of IIR filters also depends on the past values of the output, and therefore can oscillate and become unstable.

>> a1=1;

>> b1=b;

>> maimp=filter(b1,a1,u);

>> maimp(1:5)

ans =

10.1321  -19.2642   10.1321         0         0

>> a1=a;

>> b1=1;

>> maimp1=filter(b1,a1,u);

## Frequency Response of a Digital Filter

# DIRECT DIGITAL FILTER DESIGN

## Using the Yulewalk function

One of the ways to describe a digital filter is “direct form I”:

It can be converted to z-domain equation and back using z transform. I have taken the shortest form with only 3 coefficients to demonstrate the process, but it can be performed on larger number of coefficients if required.

yn=b0xn+b1xn-1+a1yn-1 		Apply Z transform to both sides

Yz=b0Xz+b1Xzz-1+a1Y(z)z-1	Move all the terms containing Y(z) to the left side

Yz-a1Yzz-1=b0Xz+b1Xzz-1		Take out factors Y(z) and X(s)

Yz1-a1z-1=Xzb0+b1z-1

Hz=Y(z)X(z)=b0+b1z-11-a1z-1

Hz=zb0+b1z-a1

- in a general case

Yulewalk algorithm can approximate a frequency response curve and return coefficients of an IIR filter – b and a.

>> [bh ah]=yulewalk(8,fdes,filt);

>> sh8=freqz(bh,ah,128);

>> plot(f,abs(sh8));

Figure 6-1 shows the frequency responses of the filters of different order. It is clear what with 8 coefficients Yulewalk filter is inferior to filters designed from analogue equivalents directly, because the notch in the response does not go as deep. With 24 coefficients it is still approaching the desired response. We can conclude that Yulewalk design method is not well suited for filters with sharp frequency responses, notch filters in particular.

Figure 6-1

Examining pole-zero plot shown in Figure 6-2 we can see that 4 poles and zeros exactly cancel. The two pairs of two zeros responsible for the notch, with θ≈±18⁰, are very close to poles, which somewhat cancels their effect. Therefore it is not a very efficient notch filter. Its shortcomings are improved upon in the next section, Direct approach.

Figure 6-2

## Direct approach

We can position poles and zeros directly on the z-plane, and then work backwards to obtain filter coefficients. This method can be quite effective, because we have complete control over where we position poles and zeros.

>> z1=complex(cos(pi/10),sin(pi/10));

>> a1=21*pi/180;

>> a2=15*pi/180;

>> p1=0.95*complex(cos(a1),sin(a1));

>> p2=0.95*complex(cos(a2),sin(a2));

>> numz=poly([z1 z1 conj(z1) conj(z1)]);

>> denz=poly([p1 p2 conj(p1) conj(p2)]);

>> nfrz=freqz(nnumz,denz,128,'whole');

>> plot(f(1:64),abs(nfrz(1:64)));

We can see in Figure 6-3 that this filter has a very sharp notch at 0.1Hz as required, in fact it seems superior to previous designs as band-stop gain is less than 0.1, whereas in previous designs it is around 0.2.

Figure 6-3

If we apply the resultant filter to signal1, we can then examine the spectrum of the output signal:

>> sigd=filter(nnumz,denz,signal1);

>> plot(t,sigd);

>> plot(fr,ampl(fft(sigd)));

However, we can see that the notch is a bit too narrow. It filters out 0.1Hz signal well, but 0.1Hz component is slightly spread out in frequency spectrum. The frequencies marked in green boxes in Figure 6-4 should be filtered out too.

To widen the notch, we can try to space out the zeros slightly, but keep them on the unit circle.

Figure 6-4

After a few tries I came up with a slightly different pole-zero positioning shown in

Figure 6-5, achieved with the following code:

>> a1=22*pi/180;

>> p1=0.95*complex(cos(a1),sin(a1));

>> a2=14*pi/180;

>> p2=0.95*complex(cos(a2),sin(a2)); % poles are now 4 degrees from the middle

>> z1=complex(cos(pi/10+2*pi/180),sin(pi/10+2*pi/180));

>> z2=complex(cos(pi/10-2*pi/180),sin(pi/10-2*pi/180)); % zeros are now 2 degrees apart from each other

Figure 6-5 shows the updated pole-zero plot, and                   Figure 6-6 shows the spectrum of signal1 filtered with the new coefficients. I think it is an improvement over the original design as it filtered out more unwanted frequencies.

| Figure 6-5 | Figure 6-6 |
| --- | --- |

## Cascade approach

One of the advantages of digital filters is that the frequency responses of two or more filters connected in series can be just multiplied together to obtain total frequency response. We do not have worry about impedance matching or load matching like we do with analogue filters.  In particular, we can square a response of a particular filter to strengthen its feature, like deepen the notch in our instance. Figure 6-7 shows that apart from deepening of the notch, bandwidth of the filter increases (notch widens).

>> mod2=mod.*mod;

>> plot(f(1:64),mod(1:64),f(1:64), mod2(1:64));

Figure 6-7

# Optional section

| Figure 7-1 | Figure 7-2 |
| --- | --- |

The s-domain transfer function shown in Equation 7-1 is product of notch and low-pass filters, both squared. It is quite a tedious task to multiply out this equation by hand, but Matlab command conv can be used for that.

| G(s)=s2LC+12s2LC+s+120.4π2(s+0.4π)2 | Equation 7-1 |
| --- | --- |

To understand the conv command better, a simple example can be used:

>> conv([1 1],[1 1])

ans =

1     2     1

Which is equivalent to:

1x+1a2=1x+1a1x+1a=1x2+2xa+1a2

>> nums=conv(conv([LC 0 1],[LC 0 1]), conv([0.4*pi],[0.4*pi]));

>> dens=conv(conv([LC 1 1],[LC 1 1]), conv([1 0.4*pi],[1 0.4*pi]));

>> hs=freqs(nums,dens,w);

>> plot(f,10*log(abs(hs)));

I have plotted the low-pass, notch and combined frequency responses in              Figure 7-2.

Equation 7-2

From the knowledge gained in this report, I think there are several ways of designing a digital filter with such response:

- Use Yulewalk function to approximate the whole response

- Use bilinear transform of the whole response

- Derive differential equation of a suitable RC low-pass filter first. Then either apply z-transform to it, or use bilinear transform of its s-domain equivalent, to convert it to z-domain. Then bearing in mind that we already know the notch filter equation in terms of z, we can just multiply the two.

I will implement the last two as they are more challenging and I believe they will produce better result.

It is quite simple to perform bilinear transform in Matlab:

>> [numcd dencd]=bilinear(nums,dens,2);

>> hn=freqz(numcd,dencd,f,2);

>> plot(f(1:64),10*log(abs(hn(1:64))));

The resultant frequency response is shown in  Figure 7-3. We can see that it matches the desired response fairly well, but diverges from it at higher frequencies.

Figure 7-3

Another approach is to design a digital low-pass filter and then combine its response with the notch response. A RC low pass filter is shown in Figure 7-4.

Figure 7-4

First, we can derive the differential equation governing the filter:

yt=ut-itR

it=dy(t)dt

yt=ut-CRdy(t)dt

Applying Laplace transform:

Ys=Us-sRC×Y(s)

Ys1+sRC=U(s)

Hs=Y(s)U(s)=11+sRC

Now we can calculate the value of constant RC:

H(s)=-3dB at 0.2Hz, ω=2*0.2Hz*π=0.4*π

|jωRC|=1

RC=10.4π

We can check that we get the desired low-pass filter response using this transfer function:

>> CR=1/0.4/pi;

>> hnew=freqs([1],[CR 1],w);

>> plot(f,10*log(abs(hnew)));

>> hnew2=freqs([1],conv([CR 1],[CR 1]),w);

>> plot(f,10*log(abs(hnew2)));

Figure 7-5 shows the resultant responses. As expected, 1st order has 20dB/decade slope and -3dB point at 0.2Hz, and 2nd order has 40dB/decade slope and -6dB point at 0.2Hz.

Figure 7-5

Now we can design the corresponding digital filter. To do that, we can convert the differential equation into difference equation using backward-difference approximation:

yn=un-RCyn-y[n-1]T

yn1+RCT=un+RCTyn-1

yn=11+RCTun+11+RCTRCTy[n-1]

Now using z-transform, and taking a0=1, a1=11+RCTRCT  and b0=11+RCT:

a0×Yz=b0×Uz+a1Yzz-1

We have to be careful converting this to polynomial form in terms of z, as a coefficients come with negative sign:

>> h=freqz(b0,[1 -a0],f,2);

>> plot(f(1:64),10*log(abs(h(1:64))));

As frequency response repeats itself above half the sampling frequency, we only plot points up to 1Hz.

We can also square the response in z-domain:

>> h2=freqz(conv(b0,b0),conv([1 -a0],[1 -a0]),f,2);

>> plot(f(1:64),10*log(abs(h2(1:64))));

Figure 7-6

Now we can merge the notch and the low-pass responses by simply multiplying the transfer functions:

>> nz=conv(b0,b0);

>> dz=conv([1 -a0],[1 -a0]);

>> nzc=conv(conv(b,b),nz); % square the notch filter response

>> dzc=conv(conv(a,a),dz); % square the notch filter response

>> hc=freqz(nzc,dzc,f,2);

>> plot(f(1:64),10*log(abs(hc(1:64))));

We also square the notch filter. Figure 7-7 shows the combined response. We can see that it matches the desired response quite well indeed. This proves that the conversion methods and manipulation in z-domain were performed correctly.

Figure 7-7

It is time to filter the signal2 with our new filter!  Because the spectrum of signal2 did not show much high-frequency distortion I decided to add some, to be able to see the effect of the low-pass filter better:

>> dist=0.2*sin(2*pi*0.4*t)+0.2*sin(2*pi*0.8*t); % some distortion at 0.4 and 0.8Hz

>> sig2=signal2+dist;

>> sig2f=filter(nzc,dzc,sig2);

>> plot(t,sig2f);

>> plot(t,want2);

>> plot(fr,ampl(fft(sig2f)));

>> plot(fr,ampl(fft(sig2)));

| Figure 7-8 | Figure 7-9 |
| --- | --- |

Figure 7-9 shows that the ramp has been reconstructed quite well.           Figure 7-8 shows that 0.1Hz, and also 0.4Hz and 0.8Hz disturbances have practically vanished.

# Batch filtering

>> filt3=[mod2(1:6) zeros(1,117) mod2(6:-1:2)];

>> plot(f,filt3)

>> sig2ft=fft(sig2);

>> smoothft=sig2ft.*filt3;

>> sig2filt=abs(ifft(smoothft));

>> plot(t,sig2filt);

Figure 8-1 explains how such filter works. It effectively removes all the frequencies above 0.1Hz in a “brick-wall” fashion in the frequency domain. The filtered spectrum is then converted back to time domain.

We can see in Figure 8-2 that ramp is reconstructed reasonably well in the middle but rather bad at the beginning and end.

| Figure 8-1 | Figure 8-2 |
| --- | --- |

Batch filtering is not suitable for real time signals, as we have to have all of the data to perform it. However it can give good results once large amount of data has been collected.

# Discussion

The design procedure for a digital filter depends on what data or specifications we are given at the start. There are a few possibilities:

| Initial data | An analogue filter | A specification | A digital filter |
| --- | --- | --- | --- |
| Design steps | - Use Laplace transform to obtain s-domain transfer function. - Work out L, C and R values - Use bilinear transform to convert to z-domain equation Or - Derive differential equation and use z-transform and derivative approximation to obtain difference equation - Obtain filter coefficients | - Specify salient features of the frequency response - Use Yulewalk algorithm to approximate the response | - Arrange b and a coefficients in a polynomial form - Calculate frequency response - Manipulate as required –change gain, add additional stages etc - Implement new filter |

In majority of cases the design procedure may well combine all the design principles given above, and some others. Digital filter design may seem deceptively simple, but there are lots of things to watch out for, such as aliasing and stability issues.

# Conclusion

# References

- http://www.intmath.com/Series-binomial-theorem/4_Binomial-theorem.php

- E713 Circuit analysis and synthesis II lecture notes

- ELEC2003.3 DIGITAL FILTERS lab script N. F. Thornhill Jan 1988 Revised: A. Fernandez Aug. 1999

- Matlab help

- Digital Signal Processing – Concepts and Applications, 2nd edition, B. Mulgrew, P. Grant, J. Thompson, Palgrave Macmillan 2003