Frequency response calculations for Actimate Mini crossover low pass section, assuming constant resistive impedance of the driver of 5.3Ω.

>> C1=8.2E-6;

>> R1=1;

>> L1=0.85E-3;

>> Zs=5.3;

>> num=[5.3*C1*R1 5.3];

>> den=[Zs*L1*C1+C1*R1*L1 Zs*C1*R1+L1+0.3*C1*R1+Zs*0.3*C1 Zs+0.3];

>> freqs(num,den)

Same for high-pass section, with tweeter impedance of 4.3Ω

>> C1=6E-6;

>> C2=12E-6;

>> L1=0.13E-3;

>> num1=[4.3*L1*C1*C2 3.87*C1*C2 0 0];

>> den1=[6.5*C1*C2*L1 C2*L1+C1*L1+5.85*C1*C1 7.4*C2+0.9*C1 0];

>> freqs(num1,den1)

Size returns number of rows first, then columns, like in example:

>> a=[ 2 3; 3 4 ;5 6]

a =

2     3

3     4

5     6

>> size(a)

ans =

3     2

Converting filters to digital form using bilinear transform yields:

ys =

Column on the left is the low pass filter; on the right is high-pass filter

0.0190    0.4420

0.0518   -0.3662

0.0731   -0.1986

0.0836   -0.0391

0.0866    0.0500

0.0847    0.0696

0.0797    0.0494

0.0728    0.0195

0.0650   -0.0023

0.0571   -0.0113

0.0493   -0.0107

0.0421   -0.0059

0.0355   -0.0013

0.0296    0.0014

0.0245    0.0020

0.0200    0.0015

0.0162    0.0006

0.0131   -0.0001

0.0104   -0.0003

0.0082   -0.0003

0.0064   -0.0002

0.0049   -0.0000

0.0038    0.0000

0.0029    0.0001

0.0021    0.0000

0.0016         0

0.0011         0

0.0008         0

0.0005         0

0.0003         0

0.0002         0

0.0001         0

0.0000         0

0         0

-0.0000         0

-0.0001         0

-0.0001         0

-0.0001         0

-0.0001         0

-0.0001         0

-0.0001         0

-0.0001         0

-0.0000         0

-0.0000         0

-0.0000         0

Speaker cross-over schematic

TOP position (both switches) = active cross-over

BOTTOM – build-in passive cross-over

Microphone recording checklist

For ASIO4ALL drivers, its important that [DC80] 1 input is selected, otherwise cross-talk signal will be recorded.

- Mic phantom power ON

- Op-amp amplifier configured in differential mode with gain 10

- Op-amp board ON

- Microphone as default device

Try loopback (one of the channels directly to mic input) if no sound

Try connections

Room dimensions and modes:

Width 2.77m

Length 4.68m

Height 2.93m

| Frequency Hz | Mode | 180.1 | Tangential |
| --- | --- | --- | --- |
| 36.8 | Axial | 184 | Axial |
| 58.8 | Axial | 186.3 | Oblique |
| 62.2 | Axial | 186.5 | Axial |
| 69.3 | Tangential | 187 | Tangential |
| 72.2 | Tangential | 188.4 | Tangential |
| 73.6 | Axial | 190.1 | Tangential |
| 85.6 | Tangential | 190.6 | Oblique |
| 93.1 | Oblique | 191.1 | Tangential |
| 94.2 | Tangential | 192.7 | Tangential |
| 96.3 | Tangential | 193.1 | Tangential |
| 110.4 | Axial | 194.2 | Tangential |
| 112.9 | Oblique | 195.6 | Tangential |
| 117.6 | Axial | 198.4 | Oblique |
| 123.2 | Tangential | 199 | Oblique |
| 124.3 | Axial | 200.5 | Tangential |
| 125.1 | Tangential | 200.9 | Oblique |
| 126.7 | Tangential | 201.4 | Oblique |
| 129.7 | Tangential | 202.9 | Oblique |
| 133 | Tangential | 203.6 | Oblique |
| 137.5 | Tangential | 208 | Tangential |
| 138 | Oblique | 208.9 | Oblique |
| 138.7 | Tangential | 215.8 | Tangential |
| 139.7 | Oblique | 216.7 | Tangential |
| 142.4 | Oblique | 217.1 | Oblique |
| 144.5 | Tangential | 218.3 | Tangential |
| 147.2 | Axial | 218.9 | Oblique |
| 152 | Oblique | 220.5 | Tangential |
| 156 | Oblique | 220.8 | Axial |
| 158.5 | Tangential | 222.1 | Tangential |
| 159.8 | Tangential | 223.5 | Oblique |
| 161.3 | Tangential | 224.6 | Oblique |
| 166.3 | Tangential | 225.7 | Oblique |
| 170.2 | Oblique | 227 | Oblique |
| 171.1 | Tangential | 228 | Oblique |
| 172.8 | Oblique | 228.5 | Tangential |
| 175 | Oblique | 229.4 | Tangential |
| 176.3 | Axial | 229.7 | Tangential |
| 176.4 | Oblique | 229.7 | Oblique |

!!!! Note: Matlab displays normalised frequency 0...1 with respect to Niqyist frequency, Fs/2. Other sources present normalised frequency with respect to Fs with range 0...0.5

Design goals:

- Maximally flat magnitude of frequency response

- Constant group delay – equal time delay at all frequencies

Verification of frequency response measurement model

- RC high-pass pass filter

VoVin=sCRsCR+1

C=100nF, R=1KΩ

# Note on setting up audio drivers for Audigy SoundBlaster 2

ASIO4ALL drivers work better than Creative’s own ASIO drivers. ASIO4ALL allows recording and playback in 96KHz mode, native drivers only allow to go up to 48KHz. Also, ASIO4ALL drivers produce much smaller, positive (as opposed to acausal delay) and predictable delay of ? samples. Here’s the best configuration:

ASIO buffer size needs to be reasonable size to prevent pops and clicks from appearing on the output. In input latency compensation does not appear to make a difference. Out setting should be left at 0.

# Note on phase recovery techniques

FFT deconvolution techniques basically does not work. Quadrature phase detector does work, with error increasing with frequency (predictably). Oversampling at 96KHz should improve the results.

Custom algorithm to unwrap the phase is needed.   Diff and then cumsum method does not appear to work.

Figure 1 - unwrapped phase of sound card response vs. 64 samples delay

Filter files:

| 1 | Lp, 100nF 10K -30dB |
| --- | --- |
| 2 | Lp, 100nF 1K -30dB |
| hp 1 | Hp, 100nF 1K -30dB |
| Xover 96 lp | -30dB, cone unobstructed |
| Xover 96 lp 1 | -25dB, cone help still with a hand |
| Shelf 96 1 | -15dB 100n ¦¦ 10K +1K shelf filter |

Octave stuff

One octave is double the frequency, =f0×2≈f0×100.3 . Octave can be sub-divided in semi-octaves as follows. Say we want to have 12 semi-octave increments. Then f=f0×10(0.312×k), where k=1..12.

Swept sine stuff

xt=sinKetL-1

wt=ddtKetL-1

K=T∙w1ln⁡w2w1 , L=Tln⁡w2w1⁡

wt=w1etTln⁡⁡w2w1 – expressing angular frequency w through t

Now express t through known f

etTln⁡⁡w2w1=ww1

Take ln of both sides:

tTlnw2w1=ln2πfw1

t=ln2πfw1∙Tlnw2w1

Working out semi-octave smoothing

22000=22×100.348×N, N=?

100.348×N=2200022=1000, taking log of both sides

N=480.3log⁡(1000), N=480

Phase response of this filter:

VoVis=sC1R1R2+R2sC1R1R2+R1+R2

Inverse sine swept filter: why should inverse filter drop off at -6dB/octave?

6dB per octave means DOUBLE the amplitude (10620=2), and 4 times the power (10610=2)

P=1T0Tx2tdt=1T0Tsin2(x)dt, where T is the period of the sine wave

P(T)=12-sin2T4T

Expressing T as 1/f, and plotting P(f) against frequency, we see that power reduces by 4 times or 6dB every octave.

This corresponds to halving of amplitude every octave.

| Tw 1 | 20cm, -6dB |
| --- | --- |
| Tw 2 | I turned up know on the amplifier, so I had to re-take measurements to have consistent amplitudes |