# The development of digital amplifier with active cross-over and room equalisation

Dmitry Ignatyev

# Abstract

My project concerns with using active digital correction and cross-over filters in conjunction with class-D amplification to try to reproduce sound as accurately as possible in a room environment. Digital audio signal can be filtered in digital domain, before any amplification takes place. This can be done with high accuracy and allows for much greater flexibility than passive components. Separated high and low frequency signals can then be amplified using any method, including class D topology, and fed directly to the speakers.  But obtaining perfect sound coming out of the loudspeakers is not enough. The room creates a lot of acoustic reflections, which can significantly alter the frequency spectrum of the sound reaching the user. This can be favourable in some cases, however I’m concerned with reproducing sound as close to the original as possible.

A simulated room acoustic model will be designed and evaluated against a real room. Using such model a digital correction filter would then be designed to compensate for peaks and troughs in frequency response, and to cancel the low-frequency reflections echo.

Applying such filter to actual audio signal should measurably flatten the frequency response in a desired position and audibly improve the listening experience.

# Table of contents

Contents

Abstract	1

Table of contents	2

Introduction	2

Class D amplifiers	3

Room equalisation	3

Digital filter as active speaker cross-over	4

Putting it all together	5

Project schedule	7

References	7

# Introduction

A traditional audio system converts digital signal to analogue, amplifies it, and then feeds it through a passive cross-over inside a loudspeaker to separate high and low frequency signals to the corresponding speaker cones. This topology has a number of limitations.  Because large signal passes through the passive components in the cross-over, losses are inevitable because of non-ideal component characteristics.

Additionally, only limited tonal controls can be implemented in analogue domain, which are not adequate for room equalisation.

In alternative implementation, active cross-over in form of a digital filter can be applied to the input signal, separating it into high and low frequency components. These signals can be amplified by two separate amplifiers and fed directly to the speakers.

A digital correction filter that is independent of active-cross over filter can be introduced to correct for room acoustic contribution. Figure 1 shows a simplified diagram of the system I’m aiming to build.

Figure 1

In this project I’m aiming to simulate all three components of such system – digital filters, class-D amplifier and a model of the acoustic response of the room. Also I will simulate the feedback loops required to extract high performance from the complete system. Finally, if time allows, I’m aiming to build and evaluate a working prototype of the system using a FPGA-based device.

# Class D amplifiers

What is a digital (class D) amplifier? It is an amplifier where signal is represented as PWM (Pulse Width Modulation) waveform, which is fed directly to the output transistors. The frequency of the PWM is a lot higher that highest frequency component of audio signal, and is filtered out by passive LC filter.

Figure 2(1)

Figure 2 shows simplified topology of class D amplifier. While the theory is quite simple, real life component limitations can seriously inhibit performance of such amplifier. Some of the problems are shown in Figure 2(1). Another set of difficulties arises because large, high-speed switching currents cause radiation and ringing unless PCB is designed very carefully (8).

These factors make high performance difficult to achieve. However things can be improved dramatically by introducing feedback. When done properly, feedback can bring performance of class D amplifier inline with best analogue amplifiers (1).

Class D has a number of important advantages. PWM signal can be generated directly from digital audio signal by a processor, so no digital-to-analogue conversion takes place until after the output stage. This reduces the complexity and room for distortion. Because class D is more than 90% efficient, it runs cool on small sized heat sinks, potentially enabling it to be installed inside loudspeakers.

# Room equalisation

Modern sound reproduction systems and amplifiers achieve vanishingly low values of distortion and noise, in order of 0.02% across the audio band. However as sound is played back through speakers inside a room, distortions at the listener’s location can rise to 40% because of the room’s acoustic contribution. I feel that such huge factor in sound quality needs to be addressed in order to experience the true feel of an audio recording.

The sound created by the speakers travels through the room at the speed of 340m/s. So in a typical room it will take sound 15 milliseconds reach the wall opposite the speaker. When it reaches obstacles such as walls the sound wave is partly reflected back. At low frequencies such reflections can create a number of standing sound waves, also called modes. At higher frequencies these modes are spaced too closely to be distinguishable. The situation is complicated because sound can take multiple paths to reach the same point in space, and reflect numerous times before decaying completely. In such way the room frequency response is formed.  The frequency response is different at every point of the room, and therefore is a function of (x,y,z) coordinates.

There has been extensive research and development of room equalisation (correction) systems (2). It is a topic of vast complexity, further complicated by the fact that sound quality is far from just an objective, numerical quantity. What technically measures well may sound bad, and vice versa. Every one has their own subjective preference on what is a good sound. However generally there are a few guidelines (2)(3).

- The most efficient way to manage a room’s frequency response is by its architectural design. For the purposes of this project I consider this option unpractical.

- It is not generally beneficial to try to just try to make frequency response completely flat. Trying to fill in troughs in the response measured in one location will overwhelm the other location.

- For low frequencies, time domain compensation is desired. This means pre-distorting the signal with anti-echo, calculated based on a room impulse response, for the two to cancel at the required point.

The last point brings on the topic of acausal filters, such that output can depend not only on past and present input values, but also future input. Because we can access any part of a digital recording, or at least buffer some part of it prior to playing it, future input can be though of as input following the present input in a buffer.

# Digital filter as active speaker cross-over

Figure 3 (6)

A simple first order passive speaker cross-over is shown on the top in Figure 3. The purpose of the cross-over is to only allow correct frequencies into corresponding speakers. Failing to do so will overload the tweeter (high-frequency speaker) and distort the woofer (bass) speaker.

Active cross-over is illustrated on the bottom. Filtering is done before amplification stage in digital domain. This topology has numerous advantages over passive topology. It eliminates bulky capacitors and inductors, and allows creating filters of any order just by cascading them, without any losses. Because there are no additional components between the output of the amplifier and the speaker, output impedance seen by the speaker is lower, which improves response particularly at low frequencies.

It is also important that configuration of active filter can be changed on-the-go, allowing to dynamically apply changes. This can be useful to introduce a delay to one of the speakers relative to the other to ensure correct phase alignment.

# Putting it all together

Figure 4 shows my proposed system diagram. A test signal (a MLS sequence or a sinusoidal sweep) is applied to the system, with no correction enabled. After obtaining the resultant sound recording, impulse response is calculated. Based on it correction filters are computed, and then actual musical can be played back with correction enabled.

Figure 4

There are different approaches to room equalisation – spatially large and localised. Because acoustic response of a room is different at every point in the room, equalisation will only work in the location where measurement microphone is placed. Although there has been research into equalisation of large areas of the room (4), great number of loudspeakers was required to achieve that, around 16. Additionally, simulation presented in the work (4) only considered perfectly rectangular rooms, which is not practical.

I’m planning to create my own room acoustic simulation model in Matlab. It would model the sound propagation as vectors (rays), including parameters such as reflection and absorption coefficient of different materials, attenuation due to propagation, and a simple model of diffusion of sound. Windows to the outside can be modelled as surfaces that do not reflect any sound. Rays would be traced until they decay to certain level of reach a number of reflections.

Models employing this basic principle have been shown to produce accurate simulation results compared to real environments (5).

Such model will allow me to evaluate the correction filters easily, by implementing a model of a microphone that can be placed anywhere in the room.

I will try to find the largest spatial area that can be equalised without significant loss of accuracy when only using two loudspeakers, which is typical set-up.

To overcome highly localized nature of room equalisation, I consider modelling a headset device with two microphones on user’s head. Microphones would be located close to user’s ears; therefore they would record the exact sound a user hears. Such device could be worn during calibration to create a few “equalisation presets” for different listening positions, which could later be selected as required.

In a prototype system, this device could record the sound signal, convert it to digital format, and wirelessly transmit it to the base station, where digital correction calculation and filtering takes place.

During developing this project, I’m looking forward to increase my knowledge in the following areas:

- Design and implementation of digital filters from a specification in Matlab and on hardware platforms

- Analogue simulation in Spice-based environment

- Room acoustics model simulation in Matlab. This encompasses vector and matrix maths, acoustics and numerical methods.

Table 1 below summarized simulation and real-world prototype implementations of the system.

|  | Simulation | Prototype |
| --- | --- | --- |
| Audio or test signal | Matlab-generated audio signal | Digital audio output of a PC sound card or CD-player |
| Frequency and time correction filter | Simulink filter component | DSP processor or FPGA-based |
| Cross-over filter | Simulink filter component | DSP processor or FPGA-based |
| Class D amplifier modules | SPICE/Simulink model | High-performance hardware modules |
| Room distortion | Matlab ray trace model | Real room |
| Microphone | A part of room acoustic model | Omnidirectional microphone |
| Frequency and time correction algorithm | Matlab algorithm | C code running on a microcontroller, PC or a DSP |

Table 1

# Project schedule

Figure 5

# References

1. A DIGITAL AMPLIFICATION TECHNOLOGY TO OPTIMISE PERFORMANCE WITH HIGH RESOLUTION AUDIO. Jones, David. London : AES 31st International Conference, 2007.

2. Digital room equalisation. Gerzon, Michael. s.l. : www.audiosignal.co.uk, 1999.

3. Kuttruff, Heinrich. Room acoustics edition 5. s.l. : Spon Press, 2009.

4. Santilla, Arturo O. Spatially extended sound equalisation in rectangular rooms. s.l. : Department of Acoustic Technology, Technical University of Denmark, 2001.

5. Farina, A. Pyramid Tracing vs. Ray Tracing for the simulation of sound. s.l. : Department of Industrial Engineering, University of Parma.

6. [Online] [Cited: 28 September 2010.] http://www.prosoundweb.com/article/print/amplifier_power_master_class_not_all_ratings_are_similar.

7. Long, Marshall. Architectural Acoustics. s.l. : Elsevier Inc., 2006.

8. Balogh, Laszlo. Design And Application GuideFor High Speed MOSFET Gate Drive Circuits. s.l. : TI.